DWL Software Manual

- Turn on the power switch of DWL

- Make sure the computer/Laptop is supported with WIFI connection and turn it on.

- Choose the Wi-Fi show below (both are OK to connect, recommend to use 5G obtain a more stable connection)

(5.8G Wi-Fi  Connection，If you laptop/PC does not support this range please use HI-LINK_OF like below)
 (2.4G Wi-Fi connection)

Key:12345678

- Please unzip the installation file and run SETUP.

- Click Next

- Click Next

- Click Next

- Select Yes to allow the app from an unknown publisher to make changes to your device.

- Click Close to exit when installation is completed.

- Please click the shortcut of DWL2 on your desktop, and then right-click with your mouse.

- Click Properties at the bottom of menu.

- Select Compatibility, and tick Run this program as administrator and click OK.

( You need this authorization to be able to create a database and store related records on the computer.)

- Click the shortcut of DWL2 on the desktop, please click Log in. (Default is to log in directly)

Network settings (It is recommended to contact the network administrator for assistance with the settings. Please refer to Appendix 1 for more information.)

- Please make sure that your Wi-Fi is connected and click on "Internet Configuration" under the Configuration menu.

- Select the WIFI page. If WIFI is successfully connected, the screen will be shown as below. Enter 254 in the last box of the IP Address column and the name of controller (you may create your own name). Do not leave the name blank, then click Insert.

Once the above steps complete, there will be a record of “Number 1” shown below.

Do not leave the box blank

Do not leave the box blank

Enter 254

Enter 254

- As the WIFI is well connected, and click the Yellow icon.

- Once connected successfully, the yellow icon will turn Green.

(Connection successful)

(Connection failed. Please check that the receiver is powered on and connected, and verify step 15 for correct identification name entry.)

- Click on the Operate menu and click Sensor Tool.

After click Sensor Tool, the screen shows as below

- In the upper left corner, you can switch the display of tools. First, select the "Receiver" and then select the tool that matches the Receiver.

- After switching to the tool to be used, you can try operating it to see the displayed indicator light, as shown in the figure below:

(Red Number indicate NG，Green number indicate OK)

Wireless programming operation: (For program parameter changes, please make sure the tool is equipped with a battery and kept idle to ensure safety.)

- Clicking on "Tool Configuration" will switch to the programming page for the sensor tool.

- The program displayed on the programming page is the parameters that read from the tool and stored in the database. Before making any modifications, please read the program first. The "Write" function and program adjustment buttons can only be used after reading the program.

Important:

For safety, set the tool as non-operational status when adjusting the program

Important:

For safety, set the tool as non-operational status when adjusting the program

- By scrolling the P-Step menu and choose the P-Step you would like to program

- The boxes show yellow if the data has not been changed. The boxes will turn blue after data changed.

- Click P-Step Writing when adjustment is completed.

- Click the P-Step Adjustment at the bottom left corner of this page to adjust the P-Step sequence

- The sequence of Parameter page as shown below.

The P-Steps will run according to the sequence put in this section, from left to right. If you do not need to use this P-Step, please set it to zero. The following P-Step numbers will also be set to zero. However, there must be at least one set of available P-Steps in the program, and they cannot all be zero.

Unadjusted parameters are highlighted in yellow, and the adjusted parameters will turn blue

- The paraments of the P-Step that is adjusting will show below

- Click the reset button to reset the P-Step sequence to only execute the first program as shown in the following：

- Click Write after the adjustment is completed.

- Once the program is written into the tool, the QR code can be generated. Please ensure to read the tool’s program first and then complete the program modification before generate a QR code.

- When the QR code is available, the use can scan directly or save it as an image for future use

Operation record data page

- Click Data and Record for checking the operation data

Click record, the operation data page show as follows

- First, select the date of the operation record you would like to view, and then select the IP address of the receiver. (If necessary, you can also select the matching bit or Mac address to search for specific tools), as shown in below.：

Confirm and Click Find

The record will be listed as shown below.

- The operation record data can be exported to an Excel .CSV file by clicking the CSV button

- Multiple language for choices, the software currently supports English, Traditional Chinese, and Simplified Chinese. Please find the language setting under Configuration menu.

Appendix 1: Network settings for the BWL receiver

- Please establish a Wi-Fi connection between your computer and the DWL receiver

- The IP address of the receiving box can be found under the IPv4 DNS server location in the properties section

- Open a browser and enter the IP address of the receiver

- Enter the username: admin and password: admin, and click login.

- After accessing the web page, click on Internet Settings and select LAN. The screen page should appear as shown like below picture.

2.

2.

1.

1.

1.

1.

- Modify the relevant network settings in the red box 1 and red box 2, scroll to the bottom, and click apply.

- Once the network settings is done, please proceed to step 15 to set the IP address.

- The wired Ethernet network settings is shown below. The setup process is the same as step 15

Do not leave the box blank

Do not leave the box blank

Enter 254

Enter 254